- Contacts:

NCAR/EOL Processing and Quality Control:

Scot Loehrer (NCAR/EOL)

loehrer@ucar.edu

Original Data Source:

Dave Turner (NOAA/NSSL)

- Dataset Overview

The PECAN Fixed PISA (PECAN Integrated Sounding Array) 6 site in Hesston, Kansas (Figure 1) released radiosondes during PECAN.  The radiosondes were provided by the Atmospheric Radiation Measurement (ARM) program. This data set includes the quality controlled PECAN Fixed PISA 6 Hesston, Kansas soundings released for the PECAN field phase (2 June to 16 July 2015). A total of 105 quality-controlled, high vertical resolution (2-second) soundings are contained in the final PECAN data set.

Figure 1.  Location of the FP6 Hesston, Kansas radiosonde site.

- Project Overview

Plains Elevated Convection At Night (PECAN) was a multi-agency project designed to advance the understanding of continental, nocturnal, warm season precipitation. It focused on nocturnal convection in conditions over the central United States plains states with a stable boundary layer (SBL), a nocturnal low-level jet and the largest convective available potential energy located above the SBL. Three aircraft (University of Wyoming King Air, NOAA P-3, and NASA DC-8) as well as a large array of fixed and mobile ground instrumentation were deployed out of Kansas from 1 June to 15 July 2015. Further information on PECAN is available at the PECAN web site: https://www.eol.ucar.edu/field_projects/pecan  and information on PECAN operations is available at the PECAN Field Catalog: http://catalog.eol.ucar.edu/pecan.

- EOL Sounding Composite (ESC) File Format Description

The ESC is a columnar ASCII format consisting of 15 header records for each sounding followed by the data records with associated data quality flags.

3.1 Header Records

The header records (15 total records) contain a variety of metadata about the sounding (i.e. location, time, radiosonde type, etc).  The first five header lines contain information identifying the sounding, and have a rigidly defined form.  The following 7 header lines are used for auxiliary information and comments about the sounding, and may vary from dataset to dataset.  The last 3 header records contain header information for the data columns.  Line 13 holds the field names, line 14 the field units, and line 15 contains dashes ('-' characters) delineating the extent of the field.

The file standard header lines are as follows:

| Line | Label (padded to 35 char) | Contents |
| --- | --- | --- |
| 1 | Data Type: | Description of the type and resolution of data |
| 2 | Project ID: | Short name for the field project |
| 3 | Release Site Type/Site ID: | Description of the release site. |
| 4 | Release Location (lon,lat,alt): | Location of the release site. |
| 5 | UTC Release Time (y,m,d,h,m,s): | Time of release. |

The release location is given as:

lon (deg min), lat (deg min), lon (dec. deg), lat (dec. deg), alt (m)

Longitude in deg min is in the format: ddd mm.mm'W where ddd is the number of degrees (with leading zeros if necessary), mm.mm is the decimal number of minutes, and W represents W or E for west or east longitude, respectively. Latitude has the same format as longitude, except there are only two digits for degrees and N or S for north/south latitude.

The time of release is given as:  yyyy, mm, dd, hh:nn:ss.

Where yyyy is the year, mm is the month, dd is the day of month, and hh:nn:ss are the UTC hour, minute, and second respectively.

The seven non-standard header lines may contain any label and contents.  The labels are padded to 35 characters to match the standard header lines.  Records for this data set include the following non-standard header lines:

| Line | Label (padded to 35 char) | Contents |
| --- | --- | --- |
| 6 | Sonde Id/Sonde Type | Serial number and type of radiosonde |
| 7 | Ground Station Equipment |  |
| 8 | Ground Check Pressure |  |
| 9 | Ground Check Temperature |  |
| 10 | Ground Check Humidity |  |

3.2 Data Records

The data records each contain time from release, pressure, temperature, dew point, relative humidity, U and V wind components, wind speed and direction, ascent rate, balloon position data, altitude, and quality control flags (see the QC code description). Each data line contains 21 fields, separated by spaces, with a total width of 130 characters. The data are right-justified within the fields. All fields have one decimal place of precision, with the exception of latitude and longitude, which have three decimal places of precision. The contents and sizes of the 21 fields that appear in each data record are as follows:

| Field | Width | Format | Parameter | Units | Missing Value |
| --- | --- | --- | --- | --- | --- |
| 1 | 6 | F6.1 | Time since release | Seconds | 9999.0 |
| 2 | 6 | F6.1 | Pressure | Millibars | 9999.0 |
| 3 | 5 | F5.1 | Dry-bulb Temperature | Degrees C | 999.0 |
| 4 | 5 | F5.1 | Dew Point Temperature | Degrees C | 999.0 |
| 5 | 5 | F5.1 | Relative Humidity | Percent | 999.0 |
| 6 | 6 | F6.1 | U Wind Comp | m/s | 9999.0 |
| 7 | 6 | F6.1 | V Wind Comp | m/s | 9999.0 |
| 8 | 5 | F5.1 | Wind speed | m/s | 999.0 |
| 9 | 5 | F5.1 | Wind direction | Degrees | 999.0 |
| 10 | 5 | F5.1 | Ascent Rate | m/s | 999.0 |
| 11 | 8 | F8.3 | Longitude | Degrees | 9999.0 |
| 12 | 7 | F7.3 | Latitude | Degrees | 999.0 |
| 13 | 5 | F5.1 | Elevation Angle | Degrees | 999.0 |
| 14 | 5 | F5.1 | Azimuth Angle | Degrees | 999.0 |
| 15 | 7 | F7.1 | Altitude | Meters | 99999.0 |
| 16 | 4 | F4.1 | QC for Pressure | Code | 99.0 |
| 17 | 4 | F4.1 | QC for Temperature | Code | 99.0 |
| 18 | 4 | F4.1 | QC for Humidity | Code | 99.0 |
| 19 | 4 | F4.1 | QC for U Wind | Code | 99.0 |
| 20 | 4 | F4.1 | QC for V Wind | Code | 99.0 |
| 21 | 4 | F4.1 | QC for Ascent Rate | Code | 99.0 |

Fields 16 through 21 contain the data quality flags from the NCAR/Earth Observing Laboratory (EOL) sounding quality control procedures.  The data quality flags are defined as follows:

| Code | Description |
| --- | --- |
| 1.0 | Checked, datum seems physically reasonable. (“GOOD”) |
| 2.0 | Checked, datum seems questionable on a physical basis. (“MAYBE”) |
| 3.0 | Checked, datum seems to be in error. (“BAD”) |
| 4.0 | Checked, datum is interpolated. (“ESTIMATED”) |
| 9.0 | Checked, datum is missing. (“MISSING”) |
| 99.0 | Unchecked (QC information is “missing”.) (“UNCHECKED”) |

3.3 Data Specifics

The files contain data at two-second intervals.

The data are in files by day, so all soundings for a particular day are concatenated into a single file ordered by time.  The file naming convention is:

HESS_yyyymmdd.cls where yyyy is the year, mm is the month, and dd is the day of the month.

The FP6 Hesston, Kansas station utilized Vaisala RS92 radiosondes using GPS for windfinding during PECAN.

3.4 Sample Data

The following is a sample of the FP6 Hesston, Kansas high resolution radiosonde data in ESC format.

Data Type:                         ARM/Ascending

Project ID:                        PECAN

Release Site Type/Site ID:         FP6 Hesston, KS/HESS

Release Location (lon,lat,alt):    097 25.80'W, 38 08.40'N, -97.430, 38.140, 451.0

UTC Release Time (y,m,d,h,m,s):    2015, 06, 02, 03:03:00

Sonde Id/Sonde Type:               K1924096/Vaisala RS92

Ground Station Equipment:          MW31 V 3.64.1

Ground Check Pressure:             Ref 964.0 Sonde 963.1 Corr 0.9

Ground Check Temperature:          Ref 28.4 Sonde 28.7 Corr -0.3

Ground Check Humidity:             Ref 0.0 Sonde 3.5 Corr -3.5

/

Nominal Release Time (y,m,d,h,m,s):2015, 06, 02, 03:00:00

Time  Press  Temp  Dewpt  RH    Ucmp   Vcmp   spd   dir   Wcmp     Lon     Lat   Ele   Azi    Alt    Qp   Qt   Qrh  Qu   Qv   QdZ

sec    mb     C     C     %     m/s    m/s   m/s   deg   m/s      deg     deg   deg   deg     m    code code code code code code

------ ------ ----- ----- ----- ------ ------ ----- ----- ----- -------- ------- ----- ----- ------- ---- ---- ---- ---- ---- ----

0.0  964.0  20.3  15.9  76.0   -2.0    0.3   2.0 100.0   0.0  -97.430  38.140 999.0 999.0   451.0 99.0 99.0 99.0 99.0 99.0 99.0

2.0  963.1  19.9  14.7  71.9   -2.7    1.5   3.1 120.0   4.1 9999.000 999.000 999.0 999.0   459.1 99.0 99.0 99.0 99.0 99.0 99.0

4.0  962.0  19.8  14.4  71.2   -3.1    2.0   3.7 123.0   4.4 9999.000 999.000 999.0 999.0   468.8 99.0 99.0 99.0 99.0 99.0 99.0

3.5 Station List

| Site ID | WMO ID | Site Name | Latitude | Longitude | Elev (m) |
| --- | --- | --- | --- | --- | --- |
| FP6 | HESS | Hesston, Kansas | 38.140N | 97.430W | 451 |

4.0 Data Quality Control Procedures

- Each sounding was converted from its original format into the ESC format described above.

- Each sounding was passed through a set of automated data quality checks which included basic gross limit checks as well as rate of change checks.  This is further described in Section 4.1.

- Each sounding was visually examined utilizing the NCAR/EOL XQC sounding quality control software.  This is further described in Section 4.2.

4.1 Automated Data Quality Checks

This data set was passed through a set of automated data quality checks.  This procedure includes both gross limit checks on all parameters as well as rate-of-change checks on temperature, pressure, and ascent rate.  A version of these checks is described in Loehrer et al. (1996) and Loehrer et al. (1998).

4.1.1 Gross Limit Checks

These checks were conducted on each sounding and the data quality flags in the ESC files were adjusted as appropriate.  Only the data point under examination was flagged.  All checks also produced warning messages that specified the location of the problem and the severity of the issue.  These warning messages where then summarized statistically and examined to determine any consistent issues.

For this data set NCAR/EOL conducted the following gross limit checks.  In the table P = pressure, T = temperature, RH = relative humidity, U = U wind component, V = V wind component, B= bad, and Q = questionable.

| Parameter | Check | Parameter(s) Flagged | Flag Applied |
| --- | --- | --- | --- |
| Pressure | <0 or > 1050 | P | B |
| Altitude | < 0 or >40000 | P, T, RH | Q |
| Temperature | < -90 or > 45 | T | B |
| Dew Point | < -99.9 or > 33 > T | RH T, RH | Q Q |
| Wind Speed | < 0 or > 100 > 150 | U, V U, V | Q B |
| U Wind | < 0 or > 100 > 150 | U U | Q B |
| V Wind | < 0 or > 100 > 150 | V V | Q B |
| Wind Direction | < 0 or > 360 | U, V | B |
| Ascent Rate | < -10 or > 10 | P, T, RH | Q |

4.1.2 Vertical Consistency Checks

These checks were conducted on each sounding and the data quality flags in the ESC files were adjusted as appropriate.  These checks were started at the surface and compared each neighboring data record.  In the case of checks that ensured that the values increased/decreased as expected, only the data point under examination was flagged.  However, for the other checks, all of the data points used in the examination were flagged.  All items within the table are as previously defined.  All checks also produced warning messages that specified the location of the problem and the severity of the issue.  These warning messages where then summarized statistically and examined to determine any consistent issues.

| Parameter | Check | Parameter(s) Flagged | Flag Applied |
| --- | --- | --- | --- |
| Time | Decreasing/equal | None | None. |
| Altitude | Decreasing/equal | P, T, RH | Q |
| Pressure | Increasing/equal > 1mb/s or < -1mb/s > 2mb/s or < -2mb/s | P, T, TH P, T, TH P, T, TH | Q Q B |
| Temperature | < -15oC/km < -30oC/km > 50oC/km > 100oC/km | P, T, RH P, T, RH P, T, RH P, T, RH | Q B Q B |
| Ascent Rate | > 3m/s or < -3m/s > 5m/s or < -5m/s | P P | Q B |

4.2 Visual Data Quality Checks

Each sounding was visually examined using the NCAR/EOL XQC sounding data quality control software.  This software allows the user to view a skew-t/log-p diagram of each sounding and apply data quality flags as appropriate.  The user can zoom in on sections of soundings for detailed examination and can adjust the data quality flags for an individual point, sections of soundings, or entire soundings for each parameter individually.  The software also allows the user to override the quality flags applied by the automated procedure.

4.3 Data Quality Issues of Note

The data quality control procedures outlined above allows us to identify and, in some cases, resolve issues that could potentially impact research performed using these data sets.  The following issues were noted in these soundings.

201506150349 – From ~338 seconds (732mb) to the end of the sounding (505mb) the radiosonde has periods of very slow (~0.5 m/s) as well as very fast (up to 20 m/s) ascent rates.

201506170557 – No RH data above 824mb.

201507100157 – No wind data.

5.0 References

Loehrer, S. M., T. A. Edmands, and J. A. Moore, 1996: TOGA COARE upper-air sounding data archive: development and quality control procedures. Bull. Amer. Meteor. Soc., 77, 2651-2671.

Loehrer, S. M., S. F. Williams, and J. A. Moore, 1998: Results from UCAR/JOSS quality control of atmospheric soundings from field projects. Preprints, Tenth Symposium on Meteorological Observations and Instrumentation, Phoenix, AZ, Amer. Meteor. Soc., 1-6.