- Contacts:

NCAR/EOL Processing and Quality Control:

Scot Loehrer (NCAR/EOL)

loehrer@ucar.edu

Original Data Source:

NOAA/NWS

- Dataset Overview

The National Weather Service (NWS) routinely releases radiosondes at 00 and 12 UTC with occasional special releases at sites throughout the United States.  This data set includes the quality controlled VORTEX-SE Meso18-19 NWS soundings released at 13 sites (Figure 1) throughout the southeastern United States region during the Meso18-19 field phase (31 October 2018 to 20 April 2019). One of the stations was only available at the mandatory and significant levels (KOUN). During Meso18-19 operations, the NWS stations released soundings at 06 and 18 UTC as well as the usual 00 and 12 UTC times. A total of 4539 1-second vertical resolution and 375 mandatory/significant level soundings are contained in the final Meso18-19 data set.

Figure 1.  Location of Meso18-19 NWS radiosonde sites.  Stations with a yellow square have one second vertical resolution data and those with red square have mandatory and significant level data.

- Project Overview

The Verification of the Origins of Rotation in Tornadoes Experiment-Southeast (VORTEX-SE) is a research program to understand how environmental factors characteristic of the southeastern United States affect the formation, intensity, structure, and path of tornadoes in this region. VORTEX-SE will also determine the best methods for communicating forecast uncertainty related to these events to the public, and evaluate public response. For the Meso18-19 2018 field season a large array of 10 research radiosonde systems were deployed throughout the southeastern United States and a smaller scale set of profiling instrumentation and surface meteorological stations were deployed around northern Alabama from 1 November 2018 to 20 April 2019. Further information on VORTEX-SE is available at the VORTEX-SE web site at NCAR/EOL: https://www.eol.ucar.edu/field_projects/vortex-se and information on the Meso18-19 deployments is available at the Meso18-19  Field Catalog: http://catalog.eol.ucar.edu/meso18-19.

- EOL Sounding Composite (ESC) File Format Description

The ESC is a columnar ASCII format consisting of 15 header records for each sounding followed by the data records with associated data quality flags.

4.1 Header Records

The header records (15 total records) contain a variety of metadata about the sounding (i.e. location, time, radiosonde type, etc).  The first five header lines contain information identifying the sounding, and have a rigidly defined form.  The following 7 header lines are used for auxiliary information and comments about the sounding, and may vary from dataset to dataset.  The last 3 header records contain header information for the data columns.  Line 13 holds the field names, line 14 the field units, and line 15 contains dashes ('-' characters) delineating the extent of the field.

The file standard header lines are as follows:

| Line | Label (padded to 35 char) | Contents |
| --- | --- | --- |
| 1 | Data Type: | Description of the type and resolution of data |
| 2 | Project ID: | Short name for the field project |
| 3 | Release Site Type/Site ID: | Description of the release site. |
| 4 | Release Location (lon,lat,alt): | Location of the release site. |
| 5 | UTC Release Time (y,m,d,h,m,s): | Time of release. |

The release location is given as:

lon (deg min), lat (deg min), lon (dec. deg), lat (dec. deg), alt (m)

Longitude in deg min is in the format: ddd mm.mm'W where ddd is the number of degrees (with leading zeros if necessary), mm.mm is the decimal number of minutes, and W represents W or E for west or east longitude, respectively. Latitude has the same format as longitude, except there are only two digits for degrees and N or S for north/south latitude.

The time of release is given as:  yyyy, mm, dd, hh:nn:ss.

Where yyyy is the year, mm is the month, dd is the day of month, and hh:nn:ss are the UTC hour, minute, and second respectively.

The seven non-standard header lines may contain any label and contents.  The labels are padded to 35 characters to match the standard header lines.  Records for this data set include the following non-standard header lines:

| Line | Label (padded to 35 char) | Contents |
| --- | --- | --- |
| 6 | Ascension Number | Number sounding this year |
| 7 | Radiosonde Serial Number |  |
| 8 | Balloon Manufacturer/Type |  |
| 9 | Balloon Lot Number/Weight |  |
| 10 | Radiosonde Type/RH Sensor Type |  |
| 11 | Surface Observations |  |

4.2 Data Records

The data records each contain time from release, pressure, temperature, dew point, relative humidity, U and V wind components, wind speed and direction, ascent rate, balloon position data, altitude, and quality control flags (see the QC code description). Each data line contains 21 fields, separated by spaces, with a total width of 130 characters. The data are right-justified within the fields. All fields have one decimal place of precision, with the exception of latitude and longitude, which have three decimal places of precision. The contents and sizes of the 21 fields that appear in each data record are as follows:

| Field | Width | Format | Parameter | Units | Missing Value |
| --- | --- | --- | --- | --- | --- |
| 1 | 6 | F6.1 | Time since release | Seconds | 9999.0 |
| 2 | 6 | F6.1 | Pressure | Millibars | 9999.0 |
| 3 | 5 | F5.1 | Dry-bulb Temperature | Degrees C | 999.0 |
| 4 | 5 | F5.1 | Dew Point Temperature | Degrees C | 999.0 |
| 5 | 5 | F5.1 | Relative Humidity | Percent | 999.0 |
| 6 | 6 | F6.1 | U Wind Comp | m/s | 9999.0 |
| 7 | 6 | F6.1 | V Wind Comp | m/s | 9999.0 |
| 8 | 5 | F5.1 | Wind speed | m/s | 999.0 |
| 9 | 5 | F5.1 | Wind direction | Degrees | 999.0 |
| 10 | 5 | F5.1 | Ascent Rate | m/s | 999.0 |
| 11 | 8 | F8.3 | Longitude | Degrees | 9999.0 |
| 12 | 7 | F7.3 | Latitude | Degrees | 999.0 |
| 13 | 5 | F5.1 | Elevation Angle | Degrees | 999.0 |
| 14 | 5 | F5.1 | Azimuth Angle | Degrees | 999.0 |
| 15 | 7 | F7.1 | Geopotential Altitude | Meters | 99999.0 |
| 16 | 4 | F4.1 | QC for Pressure | Code | 99.0 |
| 17 | 4 | F4.1 | QC for Temperature | Code | 99.0 |
| 18 | 4 | F4.1 | QC for Humidity | Code | 99.0 |
| 19 | 4 | F4.1 | QC for U Wind | Code | 99.0 |
| 20 | 4 | F4.1 | QC for V Wind | Code | 99.0 |
| 21 | 4 | F4.1 | QC for Ascent Rate | Code | 99.0 |

Fields 16 through 21 contain the data quality flags from the NCAR/Earth Observing Laboratory (EOL) sounding quality control procedures.  The data quality flags are defined as follows:

| Code | Description |
| --- | --- |
| 1.0 | Checked, datum seems physically reasonable. (“GOOD”) |
| 2.0 | Checked, datum seems questionable on a physical basis. (“MAYBE”) |
| 3.0 | Checked, datum seems to be in error. (“BAD”) |
| 4.0 | Checked, datum is interpolated. (“ESTIMATED”) |
| 9.0 | Checked, datum is missing. (“MISSING”) |
| 99.0 | Unchecked (QC information is “missing”.) (“UNCHECKED”) |

4.3 Data Specifics

The files contain data at one-second intervals.

The data are in files by day, so all soundings for a particular day are concatenated into a single file ordered by time.  The file naming convention is:

NWS_yyyymmdd.cls where yyyy is the year, mm is the month, and dd is the day of the month.

The KBMX, KFFC, KFWD, KJAN, KLZK, KOHX, KSHV, and KTAE stations utilized the Lockheed Martin Sippican LMS-6 Radiosonde with the capacitance RH sensor and GPS windfinding during WE-CAN.

KOUN utilized the Lockheed Martin Sippican LMS6 with the chip thermistor, external boom mounted capacitance relative humidity sensor, and derived pressure from GPS height during WE-CAN.

The KLCH, KLIX, and KSGF stations utilized the Vaisala RS92-NGP radiosonde with twin alternatively heated Humicap capacitance RH sensors and GPS windfinding during WE-CAN.

4.4 Sample Data

The following is a sample of the NWS radiosonde data in ESC format.

Data Type:                         GTS Sounding/Ascending

Project ID:                        VORTEX-SE Meso18-19

Release Site Type/Site ID:         72357 OUN

Release Location (lon,lat,alt):    097 28.20'W, 35 13.80'N, -97.470, 35.230, 362.0

UTC Release Time (y,m,d,h,m,s):    2018, 10, 31, 23:06:00

/

/

/

/

/

/

Nominal Release Time (y,m,d,h,m,s):2018, 11, 01, 00:00:00

Time  Press  Temp  Dewpt  RH    Ucmp   Vcmp   spd   dir   Wcmp     Lon     Lat   Ele   Azi    Alt    Qp   Qt   Qrh  Qu   Qv   QdZ

sec    mb     C     C     %     m/s    m/s   m/s   deg   m/s      deg     deg   deg   deg     m    code code code code code code

------ ------ ----- ----- ----- ------ ------ ----- ----- ----- -------- ------- ----- ----- ------- ---- ---- ---- ---- ---- ----

0.0  972.0  10.2   8.0  86.2    0.5   -2.0   2.1 345.0 999.0  -97.470  35.230 999.0 999.0   362.0  2.0  2.0  2.0 99.0 99.0  9.0

9999.0 1000.0 999.0 999.0 999.0 9999.0 9999.0 999.0 999.0 999.0 9999.000 999.000 999.0 999.0   105.0  2.0  9.0  9.0  9.0  9.0  9.0

9999.0  978.2 999.0 999.0 999.0    0.5   -2.0   2.1 345.0 999.0 9999.000 999.000 999.0 999.0   304.0 99.0  9.0  9.0 99.0 99.0  9.0

4.5 Station List

| Site ID | WMO ID | Site Name | State | Latitude | Longitude | Elev (m) |
| --- | --- | --- | --- | --- | --- | --- |
| KBMX | 72230 | Birmingham | AL | 33.180 | -86.783 | 174 |
| KCRP | 72251 | Corpus Christi | TX | 27.779 | -97.505 | 15 |
| KFFC | 72215 | Peachtree City | GA | 33.356 | -84.567 | 245 |
| KFWD | 72249 | Fort Worth | TX | 32.835 | -97.298 | 195 |
| KJAN | 72235 | Jackson | MS | 32.320 | -90.080 | 91 |
| KLCH | 72240 | Lake Charles | LA | 30.126 | -93.217 | 5 |
| KLIX | 72233 | Slidell | LA | 30.338 | -89.825 | 10 |
| KLZK | 72340 | Little Rock | AR | 34.836 | -92.260 | 173 |
| KOHX | 72327 | Nashville | TN | 36.247 | -86.562 | 180 |
| KOUN | 72357 | Norman | OK | 35.230 | -97.470 | 362 |
| KSGF | 72440 | Springfield | MO | 37.236 | -93.402 | 391 |
| KSHV | 72248 | Shreveport | LA | 32.452 | -93.842 | 85 |
| KTAE | 72214 | Tallahassee | FL | 30.446 | -84.300 | 53 |

5.0 Data Quality Control Procedures

- Each sounding was converted from its original format into the ESC format described above.

- Each sounding was passed through a set of automated data quality checks which included basic gross limit checks as well as rate of change checks.  This is further described in Section 4.1.

- Each sounding was visually examined utilizing the NCAR/EOL XQC sounding quality control software.  This is further described in Section 4.2.

5.1 Automated Data Quality Checks

This data set was passed through a set of automated data quality checks.  This procedure includes both gross limit checks on all parameters as well as rate-of-change checks on temperature, pressure, and ascent rate.  A version of these checks is described in Loehrer et al. (1996) and Loehrer et al. (1998).

5.1.1 Gross Limit Checks

These checks were conducted on each sounding and the data quality flags in the ESC files were adjusted as appropriate.  Only the data point under examination was flagged.  All checks also produced warning messages that specified the location of the problem and the severity of the issue.  These warning messages where then summarized statistically and examined to determine any consistent issues.

For this data set NCAR/EOL conducted the following gross limit checks.  In the table P = pressure, T = temperature, RH = relative humidity, U = U wind component, V = V wind component, B= bad, and Q = questionable.

| Parameter | Check | Parameter(s) Flagged | Flag Applied |
| --- | --- | --- | --- |
| Pressure | <0 or > 1050 | P | B |
| Altitude | < 0 or >40000 | P, T, RH | Q |
| Temperature | < -90 or > 45 | T | B |
| Dew Point | < -99.9 or > 33 > T | RH T, RH | Q Q |
| Wind Speed | < 0 or > 100 > 150 | U, V U, V | Q B |
| U Wind | < 0 or > 100 > 150 | U U | Q B |
| V Wind | < 0 or > 100 > 150 | V V | Q B |
| Wind Direction | < 0 or > 360 | U, V | B |
| Ascent Rate | < -10 or > 10 | P, T, RH | Q |

5.1.2 Vertical Consistency Checks

These checks were conducted on each sounding and the data quality flags in the ESC files were adjusted as appropriate.  These checks were started at the surface and compared each neighboring data record.  In the case of checks that ensured that the values increased/decreased as expected, only the data point under examination was flagged.  However, for the other checks, all of the data points used in the examination were flagged.  All items within the table are as previously defined.  All checks also produced warning messages that specified the location of the problem and the severity of the issue.  These warning messages where then summarized statistically and examined to determine any consistent issues.

| Parameter | Check | Parameter(s) Flagged | Flag Applied |
| --- | --- | --- | --- |
| Time | Decreasing/equal | None | None. |
| Altitude | Decreasing/equal | P, T, RH | Q |
| Pressure | Increasing/equal > 1mb/s or < -1mb/s > 2mb/s or < -2mb/s | P, T, TH P, T, TH P, T, TH | Q Q B |
| Temperature | < -15oC/km < -30oC/km > 50oC/km > 100oC/km | P, T, RH P, T, RH P, T, RH P, T, RH | Q B Q B |
| Ascent Rate | > 3m/s or < -3m/s > 5m/s or < -5m/s | P P | Q B |

5.2 Visual Data Quality Checks

Each sounding was visually examined using the NCAR/EOL XQC sounding data quality control software.  This software allows the user to view a skew-t/log-p diagram of each sounding and apply data quality flags as appropriate.  The user can zoom in on sections of soundings for detailed examination and can adjust the data quality flags for an individual point, sections of soundings, or entire soundings for each parameter individually.  The software also allows the user to override the quality flags applied by the automated procedure.

5.3 Data Quality Issues of Note

The data quality control procedures outlined above allows us to identify and, in some cases, resolve issues that could potentially impact research performed using these data sets.  The following issues were noted in these soundings.

KBMX201810311116 – wetbulbing ~785mb

KBMX201811050509 – wetbulbing ~785mb

KBMX201812272327 – wetbulbing and questionable temp 625-350mb

KBMX201812281222 – no data above 686mb

KBMX201902201131 – no GPS/wind data

KBMX201903031702 – no data above 575mb

KBMX201903142303 – no data above 605mb

KCRP201901052301 – RH cycling up to 450mb

KCRP201903030501 – no data

KFFC201811021106 – wetbulbing ~615mb

KFFC201811201121 – wetbulbing ~650mb

KFFC201812090507 – no RH from 893-630mb

KFFC201901181116 – no GPS/wind data below 310mb

KFFC201901192317 – no data above 740mb

KFFC201901201117 – no data below 832mb

KFFC201901231115 – data below 895mb interpolated

KFFC201902101111 – wetbulbing ~815mb

KFFC201904081123 – no GPS/wind data below 164mb

KFFC201904101108 – no RH data below 735mb

KFFC201904181708 – wetbulbing ~760mb

KFFC201904190511 – no data above 920mb, no GPS/wind data

KFFC201904191134 – no data above 788mb

KFWD201810311144 – wetbulbing 778mb

KFWD201901282304 – near surface temperatures bad

KFWD201902111108 – no GPS/wind data from 815-483mb

KFWD201902230508 – no GPS/wind data below 527mb

KFWD201903021109 – no GPS/wind data below 137mb

KFWD201904071125 – wetbulbing 925mb

KFWD201904072306 – wetbulbing 815mb

KFWD201904171716 – no GPS/wind data

KFWD201904180532 – no data above 732mb

KFWD201904202323 – no GPS/wind data

KJAN201811132310 – temp below 998mb bad

KJAN201811201101 – no data above 862mb, no GPS/wind data

KJAN201901022306 – no data above 652mb

KJAN201901290705 – no data above 704mb

KJAN201902282303 – little GPS/wind data below 194mb

KJAN201903101105 – wetbulbing 678mb

KJAN201903132302 – wetbulbing 728mb

KJAN201903281101 – no GPS/wind data below 498mb

KJAN201904021108 – wind data below 987mb bad

KJAN201904161118 – no GPS/wind data below 600mb

KJAN201904211104 – wind data below 987mb bad

KLCH201903132306 – no RH data

KLCH201903261124 – little GPS/wind data below 145mb

KLIX201811011156 – no GPS/wind data below 624mb

KLIX201811021103 – no GPS/wind data from 788-561mb

KLIX201812061103 – no GPS/wind data from 656-477mb

KLIX201812302306 – wetbulbing 700mb

KLIX201903032304 – spotty GPS/wind data throughout sounding

KLIX201903281109 – no GPS/wind data

KLIX201903301110 – no GPS/wind data

KLZK201903112311 – no GPS/wind data

KLZK201903131116 – no GPS/wind data from 800-333mb

KLZK201903132350 – no RH data from 947-540mb

KLZK201903202304 – no GPS/wind data below 414mb

KLZK201903221111 – spotty GPS/wind data from 579-130mb

KLZK201904131706 – no data above 760mb, no GPS/wind data

KLZK201904181705 – no data above 675mb

KOHX201812121119 – no GPS/wind data

KOHX201812132303 – no GPS/wind data

KOHX201901041123 – no RH data from 866-650mb

KOHX201901042329 – no GPS/wind data below 178mb

KOHX201901191129 – no data above 728mb, no GPS/wind data

KOHX201901232304 – no data above 742mb

KOHX201901281125 – no GPS/wind data from 550-264mb

KOHX201902171101 – no GPS/wind data from 887-536mb

KOHX201902191805 – no RH data

KOHX201902201159 – no data above 585mb

KOHX201903031744 – no RH data below 755mb; little wind data below 478mb

KOHX201903051121 – no GPS/wind data above 937mb

KOHX201903111103 – no GPS/wind data

KOHX201903121122 – no GPS/wind data

KOHX201903162303 – no GPS/wind data below 529mb

KOHX201903191909 – temperature/RH from 222-66mb interpolated

KOHX201904042304 – no GPS/wind data from 810-567mb

KOUN201811060600 – no GPS/wind data

KOUN201812261200 – bad winds

KOUN201812262318 – bad surface RH

KSGF 201901011110 – no GPS/wind data

KSGF201903091100 – no GPS/wind data

KSGF201904011033 – no GPS/wind data from 829-306mb

KTAE 201812062304 – temperature above 200mb bad

KTAE201902061105 – temperature below 987mb interpolated

KTAE201903012301 – no GPS/wind data below 585mb

6.0 References

Loehrer, S. M., T. A. Edmands, and J. A. Moore, 1996: TOGA COARE upper-air sounding data archive: development and quality control procedures. Bull. Amer. Meteor. Soc., 77, 2651-2671.

Loehrer, S. M., S. F. Williams, and J. A. Moore, 1998: Results from UCAR/JOSS quality control of atmospheric soundings from field projects. Preprints, Tenth Symposium on Meteorological Observations and Instrumentation, Phoenix, AZ, Amer. Meteor. Soc., 1-6.